Neath Port Talbot County Borough Council

Winter Service

Operational Plan

Environment Directorate

The Winter Service Plan sets out the operational strategy for the treatment and clearance of Neath Port Talbot’s County Road Network, along with the instructed Trunk Road Network indicated by The South Wales Trunk Roads Agency.

RECORD OF CHANGES

| DATE | PAGE NO. | REASON FOR AMENDMENT | CHANGED BY |
| --- | --- | --- | --- |
| 03-12-2017 29-01-2020 | 5 All | Request/Guidance Review / Gritting Route Update | James Michael Davies Warren Hudson/ Rhianwen Bailey |

Table of Contents

1.	Introduction	3

2.	Statement of Policies and Responsibilities	3

3.	Route Planning for Carriageways	4

4.	Weather Prediction and Information	5

5.	Organisational Arrangements and Personnel	8

6.	Facilities, Plant, Vehicles and Equipment	9

7.	Information and publicity	10

8.	Emergency 'White Out' Conditions	10

9.	24 Hour Contact Numbers	11

# Introduction

Under sub-section 41 (1A) of the Highways Act 1980 the ‘highway authority are under a duty to ensure, so far as is reasonably practicable, that safe passage along a highway is not endangered by snow or ice.’

The County Borough Council in its capacity as Highway Authority within Neath Port Talbot is responsible for winter service activities on all adopted highways in Neath, Port Talbot and the Upper Lliw Valley (except as stated below).

The South Wales Trunk Road Agency (SWTRA) acts as Agent Highway Authority for the Welsh Government and is responsible for the M4 motorway, A465 and sections of the A48 Trunk Roads in the County Borough including the sections of motorway in each of the adjoining unitary authorities of Carmarthenshire, Swansea and Bridgend. Neath Port Talbot CBC acts as a term contractor for SWTRA in this respect.  SWTRAs Adverse Weather Plan sets out the Welsh Government’s policy and objectives in the context of local service delivery on trunk roads in accordance with Welsh Government’s Trunk Road Maintenance Manual 2015 section 2.2.22, Winter and Adverse Weather Service, Version 4 - 29/7/14.

Appendix H to Well Maintained Highways, the code of practice for highways maintenance management covers winter service issues and provides guidance to highway authorities on winter service operations.  This code and periodic revisions to it are considered in collaboration with other Welsh Council’s via CSS Wales as part of maintaining this authority’s plan.

All aspects of this Plan are reviewed periodically in consultation with key stakeholders, to consider any changing circumstances and requests for additional services. The aim is to continually strive to improve upon the effectiveness of the winter service provided.

# Statement of Policies and Responsibilities

The Director of Environment has devolved the planning of the winter service activity to the Highways and Drainage Services Section within Streetcare Services, whilst the execution of winter service activity is undertaken by Streetcare Services in general.

Under instruction from SWTRA, Streetcare Services are also responsible for winter service operations on the M4 between Pyle (Junction 37) in the east and Pont Abraham (Junction 49) in the west, the short length of A48 between Baglan in the east and Earlswood in the west and the A465 between Llandarcy and the County Borough boundary at Glynneath.

Prior to the winter season there is liaison with all neighbouring unitary authorities (Carmarthenshire CC, Powys CC, Rhondda Cynon Taf CBC, CC Swansea and Bridgend CBC) to confirm cross-boundary salting arrangements and also with South Wales Police to confirm contact arrangements in case of emergencies. The cross border salting arrangements can be found in the appendices of this document.

The winter 'risk period' is considered to run from mid-October to mid-March.

At evenings and weekends (out of hours), salting arrangements and any changes to planned action are co-ordinated by the Standby Officer.

# Route Planning for Carriageways

Given in the appendices are the identified carriageway routes for pre-treatment salting. These routes have been compiled in accordance with the Winter Service Review reported to the Council’s Environment and Highways Cabinet Board in July 2014. There has since been minor amendments made to the routes to accommodate newly adopted highways & Health and Safety motives.

Pre-treatment salting is carried out such that the identified routes are treated prior to the forecast time for the onset of frost or icy conditions. Furthermore, if icy conditions persist the routes then continue to be treated as and when deemed necessary. All precautionary treatment routes take approximately 3 hours 30 minutes to complete. Treatment is also carried out on other routes as demand and the level of resources permit with the prioritisation as detailed below. Any post-treatment salting is carried out on a rolling basis using salting vehicles as well as hand salting, again as far as is practical and resources permit. Prioritisation will take into account gradients and usage etc. which will similarly apply during snow conditions where there is a requirement for snow clearing.

- Motorways and trunk roads [as directed by SWTRA];

- 'A' and 'B' roads carrying industrial traffic, access to hospitals, fire    stations and emergency centres;

- Other 'C' and Class III roads connecting villages to (i) and (ii) above.

- Access to key sources of fuel and food;

- Bus routes (including school bus routes). Industrial estate roads;

- Through routes in built up areas;

- "Short cut" routes;

- Lanes serving isolated farms;

- Any other routes deemed necessary at the time.

No pre-treatment salting is carried out on footways or cycle routes. However, when icy conditions persist post-treatment salting is carried out on high usage footways including town centre pedestrian areas and strategic footways near schools, hospitals etc.

In appendix 7 is a schedule of locations, listed geographically, that have been identified as being appropriate for consideration for post-treatment salting of carriageways and / or footways where appropriate.

Attached in appendix 1 is a schedule of plant, vehicles and equipment expected to be available for deployment.

With respect to salt bins, an inventory is maintained by the Network Management Section showing the locations of all the salt bins in the County Borough and these are maintained and stocked when necessary by Streetcare Service teams on a rolling basis throughout the winter season.

Salting over level crossings can cause signal faults. There is one level crossing that forms part of the precautionary salting network, situated in Gwaun-Cae-Gurwen. Treatment will be undertaken up to the stop line either side of the level crossing as per Network Rail guidance.

# Weather Prediction and Information

In order to monitor weather conditions, the Authority has five weather stations, consisting of two fixed road outstations at Abernant and Banwen, allowing data to be received into Campbell’s ‘Konect Global’ Data Service.  In addition, the Council has installed Campbell Scientific remote sensor units, with supporting deep and surface temperature probes, based at Melin Court, Afan Valley and Kenfig Industrial Estate.  These weather stations provide live weather information to the Authority’s forecaster.

A detailed daily forecast is received from the County Borough Council's Meteorological Contractor, Metdesk through the winter period. Metdesk’s details can be found below:

MetDesk

3 Station Approach

Wendover

Buckinghamshire

HP22 6BN

weather@metdesk.com

The forecast is displayed on MetDesk’s website by 1300 hours each day and the Council also receives advance warning of adverse weather conditions in order to initiate any contingency plans.

A decision as to whether precautionary salting is necessary and, if so, what arrangements are to be put in place is then made by the designated officer who then contacts the on-call service crews accordingly. On the weekends and public holidays the duty standby officer makes the decision whether or not to carry out precautionary salting along with any consequent arrangements. Decision making training is undertaken by all officers who are required to make a decision on precautionary salting. The guidance utilised by decision making officers, which determines whether or not precautionary treatment is required can be found in the Appendices.

The Action is then circulated to the distribution list below via e-mail.

Organisation

Ambulance Control

Bridgend CBC

Carmarthenshire CC

CC Swansea

CCTV NPTCBC

Concierge, The Quays

Customer Service Centre

Director of Environment

Head of Streetcare

Highways and Drainage Service Manager

Network and Programme Manager

Senior System Support Officer

Fleet Manager

Workshop Manager, Fleet

Business Support Officer, Engineering and Transport

Dwr Cymru

Exactrak

Fire Control

Metdesk (weather)

Powys CC

Rhondda Cynon Taff CBC Standby

South Wales Police

Standby1

SWTRA

# Organisational Arrangements and Personnel

The Standby Officer rota and the service standby rota are compiled and maintained by the Highways and Drainage Section within Streetcare Services. The Standby Officer and the service operatives are on standby on a weekly basis. Copies of the rotas are issued to staff as appropriate.

The priorities for dealing with icy conditions on County Borough Roads are as follow:

- Based on the weather forecast provided by Metdesk the Highway and Drainage Section instructs precautionary salting on some or all of the routes designated in this plan. In the case of a prolonged overnight frost a second run or early morning run may be required.

- After normal office hours and at weekends, the Standby Duty Officer decides on, or amends, the action to be taken based on the latest weather forecast information available.

- The routes listed in this plan cover 35% of the county road network.

The Council has 6 county spreader vehicles available, one of which is a reserve vehicle covering the 4 county winter service routes, and the sixth is dedicated for use on the A465.  There are also two snow blowers in the fleet which are stationed at the salt barn in the Service Response Centre, Baglan for county routes.

The anticipated deployment of plant and operatives is indicated in the Appendices.

Winter service on the motorway and trunk roads is carried out in accordance with the South Wales Trunk Road Agency Adverse Weather Plan and in accordance with Welsh Government’s Statement of Service and Code of Practice.

The Welsh Local Government Association recommends that salt stocks are replenished prior to the winter season to levels of 1.5 times the average salt usage over the past 6 years. The authority recognises the importance of these minimum required levels, which resulted in the recent construction of a new salt barn located at the Service Response Centre, The Quays. The Highways and Drainage Section manages a planned storage capacity of 7500 tonne of rock salt to be used on County roads.

The rock salt is currently obtained from:

Irish Salt Sales Co.

Fort Road

Carrickfergus

Co. Antrim N. Ireland

BT38 9BT

Rock salt for use on SWTRA routes is ordered directly by SWTRA and stored at Ynysforgan, Margam and Pont Abraham Depots.

All drivers and supervisors involved in Winter Service operations are trained to appropriate City and Guilds Vocational Qualifications. Refresher training relating to loading and driving is also carried out for all operatives annually prior to the winter season. Risk assessments are also distributed to all operatives identifying the risk and appropriate control measures associated with all operations involved in the winter service operation. A copy of relevant health and safety information is also located in the salt barn at the SRC. Due to issues associated with lone person working a mate is deployed on the treatment route that runs over the Bwlch Mountain. Drivers in general operate with mates during snow ploughing.

# Facilities, Plant, Vehicles and Equipment

All garaging, servicing and maintenance arrangements are carried out by Fleet Services working out of Tregelles Court, Neath Abbey and fitters are available on a standby basis. The Fleet Manager is responsible for administering any vehicle contract hire arrangements deemed necessary.

Calibration of salt spreading equipment is carried out by ECON, the manufacturers of the vehicles involved in the service operation.

Fuel stocks are kept and maintained at The Quays, Briton Ferry and Tregelles Court, Neath Abbey.

# Information and publicity

The South Wales Police Authority act as a liaison body for the passing of information to broadcasters relating to hazardous road conditions in the County Borough maintenance area.

The main hazards reported are:-

(i) Fog

(ii) Severe frosting

(iii) Snow

(iv) Obstructions e.g. wind, flooding and rockslides.

All information is passed to South Wales Police Headquarters for the attention of the Operations Room Duty Inspector.

To avoid duplication, information is only transmitted by the Head of Streetcare or nominated representative. All information must be clear and precise, as it is intended for broadcast word for word to avoid editorial problems.

# Emergency 'White Out' Conditions

In ‘white out’ conditions, service operations will continue on both Motorway and Trunk Roads as directed by SWTRA.

Following routine salting County Road vehicles will revert to principal road salting and ploughing at the onset of snow and will continue until the situation is under control. In a snow clearance situation more resource can be drawn from across Streetcare Services to assist operations. Ultimately, others may be mobilised from construction and private industry.

Remaining resources will cover other roads including known ‘black spots’ and hand spreading of salt will be carried out as set out in this plan, as resources permit and as directed by the Duty Officer dependent on demand and information to hand.

Contracted resources will be directed by a designated officer who will keep a log of hours worked etc.

'Road Closed' signs will be kept at the ready for immediate placement.

The SRC at The Quays will be staffed continually and a log of reports from supervisors, drivers, police and the public etc. will be kept.

All calls received will be diverted from the contact centre to the staff at the SRC coordinating service operations.

Where possible, a wall map of a suitable scale will be kept up to date to show conditions of roads within each area so that a quick, visible appraisal of the current situation is to hand, thus enabling the senior officer on duty to divert resources to where it is most needed. Supervisors and operatives will report back to the Service Response Centre on the severity of conditions regularly and ask for further instructions.

# 24 Hour Contact Numbers

Neath Port Talbot County Borough Council - 01639 686868

South Wales Trunk Road Agency - 0845 602 6020

Powys County Council - 0845 054 4847

Carmarthenshire County Council - 01267 222288

Bridgend County Borough Council - 01656 642541

City and County of Swansea - 01792 841601

Rhondda Cynon Taf County Borough Council - 01443 425001

Website

www.npt.gov.uk

Social Media

www.facebook.com/NeathPortTalbotCBC

Table of Contents

11.1 - APPENDIX 1 - Plant / Vehicle Deployment for precautionary salting	13

11.2 - APPENDIX 2 - Human Resources Deployment for precautionary salting	14

11.3 - APPENDIX 3 – Cross Boundary Spreading Arrangements	15

11.3.1 - Bridgend CBC	15

11.3.2 - Powys CC	15

11.3.3 - Carmarthenshire CC	16

11.3.4 - Rhondda Cynon Taf CBC	16

11.3.5 - City and County of Swansea	16

11.4 - APPENDIX 4 - Precautionary Salting Routes - Highways	17

11.4.1 - High Route 1	17

11.4.2 - Low Route 1	22

11.4.3 - High Route 2	27

11.4.4 - Low Route 2	32

11.5 - APPENDIX 5 – Precautionary Treatment Decision Making Guidance	36

11.6 - APPENDIX 6 – Treatment Matrix Guidance	37

11.7 – APPENDIX 7 – Post Treatment Salting	39

| ## 11.1 - APPENDIX 1 - Plant / Vehicle Deployment for precautionary salting DEPOT | ROUTE | WELSH GOVERNMENT SPREADERS | NPTCBC SPREADERS | SNOW BLOWERS | LOADER |
| --- | --- | --- | --- | --- | --- |
| Ynysforgan | Route - YG01 | CE52 NHN Scania |  | OBO 515X (W.O.) Schmit | CE52 OCG |
|  | Route - YG02 | X605 CTX Scania |  |  |  |
|  | Route - YG03 | CN51 FBG Scania |  |  |  |
|  | Route - YG04 |  | CN60 EGF |  |  |
|  | Reserve | CE52 NHO Scania |  |  |  |
| Service Response Centre | High Route1 |  | CF15 LCZ | Snowcutter attachment for JCB | Telehandler |
|  | Low Route 1 |  | CN11 BBO |  |  |
|  | High Route 2 |  | CN11 BBZ |  |  |
|  | Low Route 2 |  | CN11 BBJ | NCY 678X (C.B.C.) Rolba |  |
|  | Reserve |  | CN11 BBU |  |  |

| HUMAN RESOURCES | WELSH GOVERNMENT | NPT COUNTY BOROUGH COUNCIL | TOTAL |
| --- | --- | --- | --- |
|  | YNYSFORGAN | SRC |  |
| DRIVER | 4 | 4 | 8 |
| MATE |  | 1 | 1 |
| LOADER |  | 1 | 1 |
| HOPPER LOADER | 1 |  | 1 |

## 11.2 - APPENDIX 2 - Human Resources Deployment for precautionary salting

## 11.3 - APPENDIX 3 – Cross Boundary Spreading Arrangements

### 11.3.1 - Bridgend CBC

B4282 Bryn to Maesteg

NPTCBC to treat and turn at county boundary.

B4283 Water Street, Margam

NPTCBC to treat and turn at county boundary.

A48 Margam to Pyle

NPTCBC to treat and turn at county boundary.

A4107 Bwlch mountain road Abergwynfi to Nantymoel

Bridgend CBC to treat beyond county boundary to turn at picnic area.

### 11.3.2 - Powys CC

Rhos Common – Crynant to Glanrhyd road

NPTCBC to treat approximately a length of 100m out of the county boundary to the junction with the road entering Crynant forestry (forestry commission land).

A4109 Onllwyn to Coelbren

Powys CC to treat beyond county boundary to junction with A4109. Turn right and free run to junction with C0197. Turn right onto C0197 and commence salting to county boundary.

Pontneathvaughan

NPTCBC to treat B4242 Pontneathvaughan Road beyond county boundary and turn left to treat into Ystradfellte Road and turn at entrance to Glynneath Golf Club.

A4067 Ystalfera by-pass

NPTCBC to treat beyond county boundary to Gurnos roundabout.

B4599 to Gurnos Road and Ystalfera

NPTCBC to treat from junction of B4599 and Gurnos Road (Tesco R/A) to county boundary.

### 11.3.3 - Carmarthenshire CC

A4068 Cwmllynfell to GCG

NPTCBC treat Gwilym Road to county boundary at Cefn Bryn Brain before travelling to recommence saltting at county boundary by the Brynamman Rugby Club.

A474 GCG to Garnant

NPTCBC treat and turn at county boundary.

### 11.3.4 - Rhondda Cynon Taf CBC

C239 Pontwalby to Cefn Rhigos

RCTCBC to treat beyond boundary to turn in lay-by in NPT.

### 11.3.5 - City and County of Swansea

A483 Fabian Way

NPTCBC to treat westbound carriageway beyond county boundary to Docks entrance, turn left and treat around bend and past Gower Chemicals then return and treat eastbound carriageway back to and beyond county boundary and into NPT.

## 11.4 - APPENDIX 4 - Precautionary Salting Routes - Highways

### 11.4.1 - High Route 1

(I.e. Cymmer, Croeserw, Glyncorrwg, Abercregan, Gwynfi, Pontrhydyfen, Tonmawr, A4107, Cwmavon, Bryn, Goytre, Cimla, Tonna, Resolven.)

A

Abercregan Road C320

Afan Road A4107

Afan Valley Road A4107

Afan Valley Road B4287

Alban Terrace

Avon Street C250

Avondale Terrace C250

B

Bridge Street C250

Bryngyrnos Street B4282

Brynheulog

Brynna Road

Brytwn Road A4107

C

Castle Drive

Cavell Street

Cefncoed Road

Chapel Terrace

Church Street

Cimla Road B4287

Commercial Road B4287

Commercial Street

Coronantion Avenue

Craig y Fan Terrace C320

Cwmclais Road

Cymmer Road C250

D

Dan y Coed

Danyffynon A4107

Depot Road B4286

Dulais Fach Road B4434

Dunraven Street

Dyffryn Road

E

Easton Avenue

G

Glannant Way

Glyncorrwg Link Road

Gnoll Park Road B4434

Graig Road A4107

Greenwood Drive (P.O)

Goytre Road

H

Henfaes Road B4434

Heol Camlas

Heol Cwm Mawr

Heol Jiwbili

Heol Mabon

Heol Tewgoed

Heol y Bwlch

Heol y Glyn C250

Heol y Parc

High Street A4107

High Street B4286

J

Jersey Road C260

Jersey Terrace

John's Terrace

L

Llantwit Road B4434

Lletty Dafyod B4434

Llettyharri A4107

Lloyds Terrace A4107

London Row (P.O)

M

Mackworth Drive

Maesteg Road A4063

Maesteg Road B4282

Maesteg Road C250

Maesteg Road Lane Link

Main Road B4287

N

Neath Road B4434

P

Park Street B4434

Park Street (P.O)

Pen - y- Cae Road A4107

Pen Twyn Road A4107

Pen y Bryn

Pen y Mynydd

Prosser’s Terrace C320

Q

Queen Street B4287

R

Railway Terrace C320

S

Salem Road B4286

School Road

South Avenue

St Anne’s Terrace B4434

Station Road A4107

Station Road C260

T

Tabernacle Terrace

Tanygroes Place

Tonclwyda B4434

Tonna Uchaf B4434

Tonmawr Road C179

Ty'r Owen Row B4286

V

Varteg Road

Varteg Row

Victoria Gardens B4434

Y

Y Bellan

Ynysgwas Hill

### 11.4.2 - Low Route 1

(I.e. Port Talbot, Taibach, Margam, Aberavon, Baglan, Briton Ferry, Neath, Cimla.)

A

Abbey Road A4107

Afan Way

Alford Street

Alfred Street

B

Baglan Way

Briton Ferry Road A474

Brunel Way

Brynawel

Brynhyfryd Road

Burnside

Bus Station (P.T and Neath)

C

Cae Rhys Ddu

Cimla Crescent

Cook Rees Avenue

Commercial Road A48

Croft Road B4434

Cryddan Road

Cwmavon Road B4286

D

Dan Y Graig Road

Dalton Road

Dan y Bryn Road

Dinas Baglan Road A48

Dyffryn Road

E

Eastland Road B4434

F

Fair lawns

Forge Road

G

Giant’s Grave Road

Glan Hafren

Green Street

Groves Road

H

Harbour Way A4241

Heilbronn Way A48

High Street B4286

Hill Side

K

Kingdom Own Road (P.O)

L

Llansawel Crescent

Lodge Drive

London Road

London Row

M

Maes Ty Canol

Margam Road A48

Margam to Pyle Road A48

Moorland (P.O)

N

Neath Road A474

Neath Flyover A474

New Street

O

Old Road (Baglan)

Old Road (Briton Ferry)

Orchard Street

P

Pantyrheol A474

Pentwyn Baglan Road A48

Pentyla Baglan Road A48

Peripheral Distrbutor Road A4241

Princess Margaret Way C290

Prior Street

Prince of Wales Drive

Q

Quays Car Park

Queen Street

R

Riverside Drive B4434

S

Sarnfan Baglan Road A48

School Road

Seaway Parade A4241

Shelone Terrace C

SRC Car Park

St Illtyds Drive

Sunny Mount

T

Talbot Road A48

Tanygroes Place

Tanygroes Street

The Parade

The Ropewalk

Tyn y Twr

Ty'r Owen Row B4286

V

Victoria Gardens B4434

Victoria Road C290

W

Water Street A4241

Water Street B4283

West End Access Road

Willow Grove

Willow Way

Wind Street

Windsor Road B4434

Y

Ynysmaerdy Road

Ynys Street

### 11.4.3 - High Route 2

(I.e. Rhydyfro, GCG, Tairgwaith, Brynamman, Cwmllynfell, Rhiwfawr, Ystalyfera, Crynant, Severn Sisters, Banwen, Glynneath, Pontneddfechan, Cwmgwrach, B4242, Cilfrew.)

A

Aberdale Road

Alltygrug Farm Road

Alltygrug Road

Amman Road A4069

B

Blaengwrach Road B4242

Brecon Road B4603

Bridge Street C175

Bron yr Grug

Brook Terrace

Bryn Road C174

Brynhyfryd Terrace A4109

Brynamman Road A4069

C

Cannon Street A4069

Chain Walk B4242

Church Road A4109

Church Road C154

Coedffaldau C174

Commercial Street

Commercial Road A474

Commercial Street C154

Crynant Common Road C153

Cyfyng Road C154

D

Dulais Road A4109

E

Edward Street

Empire Avenue

F

Fothergrill Road (P.O)

G

Gelligron Road A474

Glyn Neath Road B4242

Golwg Y Bryn A4109

Graig Road (P.O) A474

Gurnos Road B4599

Gwilym Road (P.O) A4068

H

Heol Cae Gurwen A474

Heol Gaer A4109

Heol Heddwch

Heol Hen

Heol y Goedlan C175

Heol y Gors A474

Heol Wenallt

Heol Y Glyn

High Street (P.O)

High Street A4109

I

Inervalley Road A4109

L

Launderdale Road (P.O)

Llwyncelyn Road (P.O)

M

Main Road

Main Road A4109

Main Road A4230

Main Road C182

Merthyr Road

N

Neath Road A4109

Neath Road B4242

New Road B4603

New Road (Godre'r Graig)

O

Oakview C182

Onllwyn Road A4109

P

Parish Road

Park Avenue B4242

Park Street A4069

Penscynor

Pen yr Allt (P.O)

Penywern Road

Pontardawe Road A474

Pontneathvaughan Road B4242

R

Rhiw Road C175

Rhiwfawr Road C175

Roman Road

S

School Road

St Davids Road

W

Waun Penlan

Wembley Avenue A4109

Wern Road C154

Y

Ynysgerwen Avenue B4242

Ynysydarren Road B4599

### 11.4.4 - Low Route 2

(I.e. Fabian Way, Llandarcy, Jersey Marine, Skewen, Neath Abbey, Bryncoch, Rhos, Alltwen, Trebanos, Pontardawe, Cilmaengwyn, God-re-graig, Ynysmeaudw.)

A

Alltywern

Alltwen Hill C155

Alloy Industrial Estate

Allt y cham Drive (P.O)

Arthur Terrace

Ashleigh Terrace B4290

B

Brecon Road B4603

Bryn Llewelyn

Brynawel

Burrows Road B4290

C

Cefnllan Road (P.O)

Cilmaengwyn

Cwmdu Road

Cadoxton Road A474

Christopher Road

Crymlyn Road C110

Cloda Avenue (P.O)

D

Darcy Business Park

Delffordd

Drumau Road

Dwr Y Felin Road

F

Fabian Way A483

Farmers Road C163

Ffordd Amazon

Ffordd Parc Ynysderw

Ffordd Cwmtawe

G

Gelligron Road A474

Gnoll Road

Glais Road C155

Graig Road A474

Grove Road

Gwyn's Place C155

Gwyn's Street C155

H

Herbert Street Bypass A474

Herbert Street C360

High Street B4603

Holy Street C360

Heol y Nant

Heol Escyn

Heol Penlan

J

James Street

L

Longford Road C163

M

Maesycoed Road

Main Road C163

N

Neath Road

New Road B4603

New Road A474

New Road A4230

New Road B4290

Neath Abbey Road

O

Old Road B4290

P

Park Avenue A4230

Penyheol B4290

Penywern Road

Pen Yr Alley Avenue

Pen Yr Alltwen

Pontardawe Road B4603

R

Rhyd Hir

S

Siding Terrace A4230

St. Johns Terrace

Stanley Road

Swansea Road (P.O) A474

T

Ty’n Yr Heol Road

W

Wern Road C11

Winifred Road

Y

Ynys-Meudwy Road B4603

Ynys y Mond Road C155

## 11.5 - APPENDIX 5 – Precautionary Treatment Decision Making Guidance

| NPT - Precautionary Treatment Decision Guide |  |  |  |  |
| --- | --- | --- | --- | --- |
| Road Surface Temperature | Precipitation | Predicted Road Conditions |  |  |
|  |  | Wet | Wet Patches | Dry |
| May fall below 1°C | No rain | Salt before frost | Salt before frost | No action likely, monitor weather |
|  | No hoar frost |  |  |  |
|  | No fog |  |  |  |
| Expected to fall below 1°C | No rain |  |  |  |
|  | No hoar frost |  |  |  |
|  | No fog |  |  |  |
|  | Expected hoar frost |  | Salt before frost |  |
|  | Expected fog |  |  |  |
|  | Expected rain BEFORE freezing | Salt after rain stops |  |  |
|  | Expected rain DURING freezing | Salt before frost, as required during rain and after rain stops |  |  |
|  | Possible rain | Salt before frost |  | Monitor weather conditions |
|  | Possible hoar frost |  |  |  |
|  | Possible fog |  |  |  |
| Expected snow |  | Salt before snow fall |  |  |
| The decision to undertake precautionary treatments should be, if appropriate, adjusted to take account of residual salt. |  |  |  |  |
| All decisions should be evidence based, recorded and require continuous monitoring and review. |  |  |  |  |
| Decision on treatment timing should account for traffic and road surface wetness at time of treatment and after, as well as forecast conditions. |  |  |  |  |

## 11.6 - APPENDIX 6 – Treatment Matrix Guidance

## 11.7 – APPENDIX 7 – Post Treatment Salting